HS-10 Barcode Scanner

User Manual

V1.1.1

Disclaimers

Please read all the contents of this manual carefully before using the products described in this manual to ensure safe and effective use of the products. It is recommended that this manual be kept in a safe place for the next time you use it for reference.

Warnings

Please do not disassemble the product or tear the seal label on the product by yourself, otherwise our company will not be responsible for warranty or replacement of the product.

The pictures in this manual are for reference only. If there is any discrepancy between individual pictures and actual products, please refer to the actual products. For product improvement and update, the company reserves the right to modify the document at any time without notice.

All information contained in this manual is protected by copyright and all rights are reserved. No unit or individual may make any kind of excerpt, copy or bundle with other products or sell this document in whole or in part in any way or for any reason without written permission.

Our company has the final right to interpret this manual.

Service Information

If you need more technical support, please call or email us and we will be happy to serve you.

Version Record

| Instruction manual version number | Version Description | Release Date |
| --- | --- | --- |
| V1.0 | Initial Version | 2021.12.27 |

Catalog

Preface	1

Introduction	1

Chapter outline	1

Brochure Legend	1

Chapter 1 About HS10	2

Introduction	2

Main Features	2

Open the package	2

Communication Port	3

Power on, power off, standby and restart	3

Maintenance and Care	3

Code Reading Tips	4

Chapter 2 System Setup	5

Introduction	5

Setting the logo	5

Use the setup code	5

Restore factory default settings	6

Read the version number	6

Product User Settings	6

Sound settings	7

Volume settings	7

Set the beep switch	7

Power On Beep Switch	8

Read success beep switch	8

Chapter 3 Reading Mode Settings	9

Level trigger mode	9

Continuous mode (default)	9

Pulse trigger mode	9

Repeat read switch	10

One read timeout	10

Chapter 4 Communication Setup	11

Introduction	11

Communication settings	11

System Upgrade	11

USB HID upgrade steps	11

RS-232 serial port upgrade procedure	13

USB Keyboard	16

USB Multi-Country Keyboard	16

USB keyboard sending speed	17

Setting the USB keyboard sending speed	18

Control characters	18

Serial communication settings	19

Serial port baud rate setting	19

Serial port parity bit setting	20

Chapter 5 Output Data Format Setting	21

Introduction	21

Custom Prefix Settings	21

Custom prefix on/off settings	21

Custom Prefixes	22

Custom Suffix Settings	22

Custom suffix on/off settings	22

Custom Suffixes	23

Code ID setting	24

Code ID output method	24

Custom Code ID	25

Terminator settings	25

Case output settings	26

Data output settings	27

Data output selection	27

前段 Pre/Post data length setting	28

Chapter 6 Decode Library Setup	29

Introduction	29

All barcodes	29

EAN8 Settings	29

EAN8 on/off setting	29

EAN8 checksum settings	30

EAN8 transcoding settings	30

Add-on code setting	30

EAN13 Settings	31

Turn on/off settings	31

Calibration settings	31

Leading code setting	31

ISSN Settings	32

ISBN settings	32

Data bit setting	32

Transcoding settings	32

UPC-A Settings	33

Checksum setting	33

Transcoding settings	33

UPC-E Settings	33

Turn on/off settings	33

校验 Checksum setting	34

Transcoding settings	34

Code 39 Settings	34

Turn on/off settings	34

Calibration settings	35

Start and stop character settings	35

Code39 Full ASCII code reading function	35

Code 39 Length Setting	36

Code 32 settings	37

Transcoding settings	37

Code 128 Settings	37

Turn on/off settings	37

Calibration settings	37

Code 128 Length Setting	38

Code 93 Settings	39

开启 Turn on/off settings	39

Calibration settings	39

Code 93 Length Setting	40

Codabar Settings	41

Turn on/off settings	41

Start and stop character settings	41

Codabar length setting	41

Crossover 25 code (ITF5) setting	42

开启 Turn on/off settings	42

Calibration settings	42

Crossover 25 yard length setting	43

Appendix	44

Appendix 1 Data code table	44

Appendix 2 Code System Support List	46

Appendix 3 Table of Control Characters	47

Appendix 4 ASCII code table	49

# Preface

## Introduction

This manual mainly provides users with detailed information on the use of HS10 and related precautions.

## Chapter outline

| Chapter 1 About HS10 |  |
| --- | --- |
| Chapter 2 System Setup | Introduce the main setting method of the scanner and the system parameters setting |
| Chapter 3 Reading Mode Settings | List the supported reading modes and provide the corresponding parameter setting codes |
| Chapter 4 Communication Setup | Describes the setting of communication parameters |
| Chapter 5 Formatting of Output Data | Introduces how to use the barcode information in the set output data format |
| Chapter 6: Decode Library Setup | Lists all the code systems supported and provides the relevant parameter setting codes |
| Appendix | Provide common setting codes and factory default parameter table, etc. |

## Brochure Legend

| ATT | Note the hint that the user needs to pay strong attention to the content here |
| --- | --- |
| Tips | Tips to help users better understand the content of the document |
| Example | Examples to help users become familiar with the operation |

# Chapter 1 About HS10

## Introduction

Our product model HS10, to which this manual applies, uses laser scanning to identify barcodes. They apply a full set of our self-developed patented technology, with powerful recognition performance, fast and flexible scanning speed.

This section will introduce the use of the product step by step with pictures, please compare the actual product you have purchased when reading the document, so that it is more conducive to your understanding of this document. This section is suitable for general users, maintenance personnel and software developers.

## Main features

* Completely self-developed, with a full set of patents, plug-and-play without installing drivers.

* With 32-bit main control chip equipped with patented software, it can smoothly decode reflective, wrinkled, blurred, and color barcodes, and can decode normally in strong light and dark environment.

## Open the package

After you open the shipping carton containing the product, take the following steps:

- Check for damage during shipment. Report damage immediately to the carrier who delivered the carton.

- Make sure the items in the carton match your order.

- Save the shipping container for later storage or shipping.

## Communication Port

The product must be connected to a host computer in order to operate. The host computer can be a PC, a POS machine or an intelligent terminal with any of the USB or RS-232 interfaces.

| USB | USB port on the host |  |
| --- | --- | --- |
| RS-232 | RS-232 interface on the host computer |  |

The scanner will be equipped with the corresponding cable according to the customer's order interface, if you need to switch to other interfaces, please consult sales to purchase the corresponding cable.

## Power on, power off, standby and restart

Power on: Connect the product to the host, and the product will automatically power on and be in working condition.

Shutdown: Remove the data cable connected to the product; remove the USB cable plugged into the host computer/the power adapter plugged into the RS-232 data.

Reboot: If the product appears to be dead or unresponsive, please turn it off and on again to achieve a reboot.

## Maintenance and Care

*The literate window must be kept clean and the supplier is exempt from warranty liability for damage caused by improper maintenance.

* Avoid abrasion or scratching of the reading window by hard and rough objects.

* Removal of smudges on the identification window with a brush.

*Please use a soft cloth to clean the window. such as eyeglass cleaning cloths and lens-specific cleaning cloths.

* Prohibit the spraying of any liquid on the identification window.

* The use of any cleaning agent other than cleaning water is prohibited.

## Code Reading Tips

If the barcode is small, you should make the barcode close to the product scanning window; if the barcode is large, you should make the barcode a little far from the product scanning window, so that it is easier to read the barcode correctly.

If the barcode is highly reflective (e.g., coated surface), you may need to tilt the barcode at an angle in order to successfully scan the barcode.

Example of sweeping code.

# Chapter 2 System Setup

## Introduction

This product is mainly used to set options and functions by reading a series of special barcodes. In this section, we will introduce in detail the options and functions available for users to set and provide the corresponding setting codes.

This method of setting up a scan is relatively straightforward and easy to understand, making it user-friendly.

## Setting Barcode

Startup settings (default) 1 2

This is the logo for the sweep configuration code function to turn on (default) barcode functionality. The logo consists of two parts. - Set the barcode part of the code - Set the name of the option or function, such as the function to turn on the configuration code function.

## Use the setup code

Read the "Startup Settings" barcode to activate the setup code function, which allows you to set up the scanner by reading the setup code.

To exit the setup code function, simply read the "Exit Setup" barcode.

| Startup settings (default) |  |
| --- | --- |
|  | Exit Settings |

## Restore factory default settings

| Restore factory settings |  |
| --- | --- |

## Read version number

Read the current device version number. If the version number data length exceeds the maximum length of the instruction, you need to send Get Instruction 1 and Get Instruction 2 respectively to get the complete version number, refer to Appendix: "Instruction Format Description" for details.

| Read version number |  |
| --- | --- |

## Product User Settings

In addition to the factory default settings, you can save the frequently used settings as user default settings.

The user default settings also contain all property settings for the scanner and the user default settings will be saved and not lost unless the current settings are re-saved as user default settings.

Save User Defaults: will save the current user default settings and overwrite the previously set user default settings.

Restore User Defaults: will cause the scanner to switch to the user default settings.

| Save user default settings |  |
| --- | --- |
|  | Restore user default settings |

## Beeper settings

Scanner sounds can be divided into: setup beep, power on beep and read success beep. This section will provide detailed instructions for setting the scanner sounds.

### Volume Settings

| Volume set to High (50%) (default) |  |
| --- | --- |
|  | Volume set to midrange (25%) |
| Volume set to bass (12.5%) |  |

### Set the beep switch

| Open (default) |  |
| --- | --- |
|  | Close |

### Power on beep switch

| Open (default) |  |
| --- | --- |
|  | Close |

### Read success beep switch

| Open (default) |  |
| --- | --- |
|  | Close |

# Chapter 3 Reading mode settings

## Level trigger mode

Enter the reading state when the trigger key pin is at low level. Press and hold the trigger key to start reading code. After reading the code successfully or releasing the trigger key, the code reading ends and the next decoding needs to re-enter the low level state.

| Level trigger mode |  |
| --- | --- |

## Continuous mode (default)

When the reading setting code is switched to this mode, it enters the continuous reading state. In this mode, the "repeat read switch" can be used to prevent the same barcode from being read more than once.

| Continuous mode (default) |  |
| --- | --- |

## Pulse trigger mode

When the reading setting code is switched to this mode, it enters the pulse-triggered reading state.

| Pulse trigger mode |  |
| --- | --- |

## Repeat read switch

The repeat read setting is only available in continuous mode.

| Allow repeat readings (default) |  |
| --- | --- |
|  | Repeat reading is not allowed |

## One read timeout

One time reading timeout time, the unit is 0.1 seconds. The default value is 30 (3 seconds), when set to 0, it means that the device is always in the reading state, and the time range can be set: 0~999.

| Short time (3s) |  |  |
| --- | --- | --- |
|  |  | In time (7s) |
| Long time(10s) |  |  |
| ATT | Valid only in "Level Trigger Mode" and "Pulse Trigger Mode". |  |

# Chapter 4 Communication Settings

## Introduction

When connecting the scanner and the host, there are various connection methods to choose from, while the default method can be set according to the actual needs of the customer, and this section will introduce the settings of the relevant communication methods in detail.

## Communication settings

| USB HID (default) |  |
| --- | --- |
|  | USB CDC serial port |
| RS-232 serial port |  |

## System Upgrade

There are two ways to upgrade the system: USB HID upgrade and RS-232 serial upgrade, USB CDC serial upgrade is not supported for now.

### USB HID upgrade steps

Take "LASER_SCANER_V1.1_20211124" version upgrade as an example.

| Serial number | Operation | Operation Feedback / Legend |
| --- | --- | --- |
| 1 | Scan the "USB HID" barcode. USB HID (default) | Automatic reboot of the device after successful code scanning |

| 2 | Scan the "System Upgrade" barcode. System Upgrade | The upgrade operation can be executed only after the successful setting beep sounds |
| --- | --- | --- |
| 3 | Open the software "HS_Update_V1.0.exe". Click on the "USB Update" menu. Click on the "Browswe" button |  |
| 4 | Select the corresponding upgrade file. Click the "Open" button |  |
| 5 | Click on the "Upadte" button |  |
| 6 | The interface displays "Update Success!", and the device is automatically reset after a successful beep. |  |
| 7 | In Window environment, open Notepad (TXT) and scan the "Read version number" barcode. Read version number | The device correctly outputs "LASER_SCANER_V1.1_20211124". Represents a successful system upgrade |

### RS-232 serial port upgrade procedure

Take "LASER_SCANER_V1.1_20211124" version upgrade as an example.

| Serial number | Operation | Operation Feedback / Legend |
| --- | --- | --- |
| 1 | Scan the "RS-232 serial" barcode. RS-232 serial port | Automatic reboot of the device after successful code scanning |
| 2 | Scan the "System Upgrade" barcode. System Upgrade | The upgrade operation can be executed only after the successful setting beep sounds |
| 3 | Open the software "HS_Update_V1.0.exe". Click on the "COM Update" menu. Click on the "Serch Port" button |  |
| 4 | "Port Select" sets the COM port. Configure the parameters as shown on the right. Click on the "Open Port" button |  |
| 5 | The "C" character appears in the interface. Click on the "Browse..." button |  |
| 6 | Select the corresponding upgrade file. Click the "Open" button |  |
| 7 | Click the "Send" button |  |
| 8 | The interface shows "File sent completed.", the device will be reset automatically after a successful beep. |  |
| 9 | Scan the "Read Version Number" barcode. Read version number | The device correctly outputs "LASER_SCANER_V1.1_20211124". Represents a successful system upgrade |

## USB keyboard

### USB multi-country keyboard

There are differences in keyboard key layout and symbols, etc. corresponding to different countries' languages, so the scanner can be virtualized into different countries' keyboard systems as needed, with the default English (US) keyboard.

| English (US) (default) |  |  |
| --- | --- | --- |
|  |  | Italian (Italy) |
| Spanish (Brazil) |  |  |
|  |  | Portuguese (Portugal) |
| Portuguese (Brazil) |  |  |
|  |  | French (France) |
| German (Austria) |  |  |
|  |  | Turkish Q |
| Turkish F |  |  |
|  |  | English (UK) |
| Japanese (Japan) |  |  |
|  |  | German (Switzerland) |
| French (Belgium) |  |  |
| ATT | Portuguese (Brazil) cannot output "?" and "/" . |  |

### USB keyboard sending speed

If there is missed data at the receiver side, the occurrence speed should be turned down, which is easy to occur at the receiver side with poor performance.

| 5ms (default) |  |
| --- | --- |
|  | 10ms |
| 20ms |  |

### Set USB keyboard sending speed

USB keyboard send speed, unit is milliseconds (ms). The default value is 5ms, the settable time range: 0ms~200ms.

| Set USB keyboard sending speed |  |
| --- | --- |
| Example | Setting the USB keyboard to send speed of 50ms can be set by reading the following barcodes in order. 1. Read "Startup Settings" 2. Read the "Set USB keyboard send speed" setting code 3. Read data code: "5" "0" (see data code table) 4. Read "Save" (see data code table) 5. Read "Exit Settings" |

### Control characters

| Off (default) |  |  |
| --- | --- | --- |
|  |  | Control + ASCII mode |
| Alt + Keypad mode |  |  |
| Tips | Specific reference to the control character table, only the data content of the barcode is converted. |  |

## Serial communication settings

| Tips | The serial data bit is 8 and the serial stop bit is 1. |
| --- | --- |

### Serial port baud rate setting

The baud rate is the number of bits per second that are transmitted for serial data communication. The baud rate used by the scanner and the data receiving host must be consistent to ensure accurate data transmission. The scanner supports the baud rates listed below, and the default is 9600bps.

| Baud rate 4800 |  |
| --- | --- |
|  | Baud rate 9600 (default) |
| Baud rate 19200 |  |
|  | Baud rate 38400 |
| Baud rate 57600 |  |
|  | Baud rate 115200 |

### Serial port parity bit setting

| No checksum (default) |  |
| --- | --- |
|  | Odd calibration |
| Even Check |  |

# Chapter 5  Data Format Setting

## Introduction

Length of custom prefix and suffix: (0~10) characters, if set to "on", "Code ID prefix", "custom prefix", "end suffix", etc. will be added before and after the decoding information. "Custom suffix", "End suffix", etc.

| ATT | The maximum number of data cache is 5, and the maximum length of individual data is 7900 characters; the data output is sequential output, and you need to wait for the first barcode output to finish before the second barcode output, and so on. |
| --- | --- |

## Custom Prefix Settings

### Custom prefix on/off settings

Custom prefixes add a user-defined string before the decoded information. For example, if you allow to add a custom prefix and set the prefix to the string "AB", after reading the barcode with the data "123", the scanner adds the string "AB" before the string "123", and the host side receives "AB123". After the barcode reading data is "123", the scanner will add the string "AB" before the string "123", and the host side will receive "AB123".

If set to "Off", the decoded information will only have the barcode data information, no prefix, and the default value is to turn off the custom prefix output.

| Enable |  |
| --- | --- |
|  | Disable (default) |
| Clear all custom prefixes |  |

### Customized prefixes

The custom prefix adds a user-defined string before the decoded information, and the output format after customization is "Custom Content + Barcode Content".

| Customized prefixes |  |
| --- | --- |
| Example | Set the custom prefix to 'a' (the hex value of a is 0x61) 1. Read "Startup Settings" 2. Read the "Custom Prefix" setting code 3. Check the ASCII code of the character "a": the ASCII code of "a" is "0x61" (see ASCII code table) 4. Read data code: "6" "1" (see data code table) 5. Read "Save" (see data code table) 6. Read "Exit Settings" |  |

## Custom Suffix Settings

### Custom suffix on/off settings

The custom suffix adds a user-defined string after the decoded information. For example, it is allowed to add a custom suffix and set the suffix to the string "AB", after reading the barcode with the data of "123", the scanner adds the string "AB" after the string "123", and the host side receives "123AB". After the barcode reading data is "123", the scanner will add the string "AB" after the string "123", and the host side will receive "123AB".

If set to "Off", only the barcode data information will be in the decoded information, no suffix, and the default value is to turn off the custom suffix output.

| Enable |  |
| --- | --- |
|  | Disable (default) |
| Clear custom suffixes |  |

### Custom Suffixes

Custom suffix adds a user-defined string after the decoded information, and the output format after customization is "barcode content + custom content".

| Custom Suffixes |  |  |
| --- | --- | --- |
| Example | Set the custom suffix to 'a' (the hex value of a is 0x61) 1. Read "Startup Settings" 2. Read the "Custom Suffix" setting code 3. Check the ASCII code of the character "a": the ASCII code of "a" is "0x61" (see ASCII code table) 4. Read data code: "6" "1" (see data code table) 5. Read "Save" (see data code table) 6. Read "Exit Settings" |  |

## Code ID setting

The user can use Code ID to identify the barcode type and the Code ID corresponding to each barcode type is customizable. The Code ID for all barcodes is 1 character.

### Code ID Output Method

Close Code ID (default): Code ID is not spliced to the read string.

Code ID prefix: The Code ID is spliced before the recognition string.

Code ID suffix: The Code ID is spliced after the recognition string.

| Disable Code ID (default) |  |  |
| --- | --- | --- |
|  |  | Code ID prefix |
| Code ID suffix |  |  |
|  |  | Clear all custom Code IDs |
| Example | Restore the Code ID of all barcodes, including Codabar, to the default value of. 1. Read "Startup Settings" Read "Clear all custom Code IDs". 3. Read "Exit Settings" |  |

### Customize Code ID

Please refer to the following example for how to modify the Code ID.

| Custom Code ID |  |  |
| --- | --- | --- |
| Example | Modify the Code ID of Codabar (Code ID:0x61) to "Y" (hexadecimal value 0x59). 1. Read "Startup Settings" 2. Read "Custom Code ID" 3. Check the Code ID of the barcode: the Code ID of "Codabar" is "0x61" (see the list of code system support) 4. Check the ASCII code of the character "Y": the ASCII code of "Y" is "0x59" (see ASCII code table) 5. Read data code: "6" "1" "5" "9" (see data table) 6. Read "Save" (see data code table) 7. Read "Exit Settings" |  |

## Terminator settings

The terminator suffix is used to mark the end of a complete piece of data information. The terminator suffix stands alone and does not participate in any other form of data formatting. The terminator suffix must be the last piece of data sent, and no additional data will be added thereafter.

The ending character can be set to carriage return, line feed, carriage return line feed, tab or ETX, and the default ending character is set to carriage return.

| No terminator |  |
| --- | --- |
|  | Enter (default) |
| Line Feed (Down) |  |
|  | Carriage return + Line feed (Enter+Down) |
| Tab |  |
|  | ETX |

## Case output settings

Case conversion for characters in a string, prefix and suffix have no effect.

| Normal output (default) |  |
| --- | --- |
|  | Convert Case |
| Convert all to Uppercase |  |
|  | Convert All to lowercase |

## Data output settings

| ATT | Note: The prefix and suffix are not processed, and the prefix and suffix are output normally. |
| --- | --- |

### Data output options

Raw data output (default): No modification is made to the literate data.

If the set length is longer than the length of the read string, the original data will be output. For example, if the length of the string "1234567890" is set to 3, the final output data will be "123".

Intermediate data output: output according to the range of data limited by the "previous data length setting" and "next data length setting", if the sum of the two length values is greater than the length of the read string, the output is empty. For example, if the length of the string "1234567890" is read, and the length of the start/end fields are set to 3 and 4 respectively, the final output data will be "456".

If the set length is longer than the length of the read string, the original data will be output. For example, if the length of the string "1234567890" is set to 3, the final output data will be "890".

Start and end data output: The output data is limited according to the data of "previous data length setting" and "next data length setting", if the sum of the two length values is greater than the length of the read string, the original data is output. For example, if the length of the string "1234567890" is read, and the length of the start/end field is set to 3 and 4 respectively, the final output data will be "1237890".

| Raw data output (default) |  |
| --- | --- |
|  | Front-end data output |
| Intermediate data output |  |
|  | Back-end data output |
| Start and end data output |  |

### Pre/Post data length setting

The default value is 1, the length should be set with the appendix, the setting range: 1 to 7900. when the length is set to 0, it is equivalent to the function is not enabled.

For example, if you want to set the "Pre-segment data length setting" to 12, steps: Read "Pre-segment data length setting"->"1"-> "2"->"Save".

| Pre-segment data length setting |  |
| --- | --- |
|  | Back-end data length setting |

# Chapter 6 Symbologies

## Introduction

Each type of barcode has its own unique properties, and the setup codes in this chapter allow you to adjust the scanner to accommodate these property changes.

## The fewer barcode types that are turned on, the faster the scanner will Scan. You can turn off the barcode types that will not be used to improve the performance of the scanner

## All barcodes

Turn off all barcodes: the scanner will only be able to Scan the setup code, all barcodes other than the setup code will not be Scan.

Open all barcodes: the scanner will Scan all barcodes that can be Scan.

| Close all barcodes |  |
| --- | --- |
|  | Turn on all barcodes |

## EAN8 Settings

### EAN8 on/off setting

| EAN8 on (default) |  |
| --- | --- |
|  | EAN8 off |

### EAN8 Check Character settings

| Enable EAN8 transmission Check Character (default) |  |
| --- | --- |
|  | Disable EAN8 transmit Check Character |

### EAN8 transcoding settings

| Convert EAN8 to EAN13 |  |
| --- | --- |
|  | Do not Convert EAN8 to EAN13 (default) |

## Add-on code setting

| Adaptive UPC/EAN add-on codes |  |
| --- | --- |
|  | Only Decode UPC/EAN Add-on codes |
| Ignore UPC/EAN add-on codes (default) |  |

## EAN13 Settings

### Turn on/off settings

| EAN13 on (default) |  |
| --- | --- |
|  | EAN13 Off |

### Calibration settings

| Enable EAN13 transmission Check Character (default) |  |
| --- | --- |
|  | Disable EAN13 transmission checks |

### Sysmte Character Setting

| Transmit EAN13 System Character (default) |  |
| --- | --- |
|  | Do not Transmit EAN13 System Character |

## ISSN Settings

| Convert EAN-13 to ISSN |  |
| --- | --- |
|  | Do not Convert EAN-13 to ISSN (default) |

## ISBN setting

### Data bit setting

| Set the ISBN data bits to 13 bits |  |
| --- | --- |
|  | Set ISBN data bits to 10 bits (default) |

### Transcoding settings

| Convert EAN13 to ISBN |  |
| --- | --- |
|  | Do not Convert EAN13 to ISBN (default) |

## UPC-A Settings

### Check Character setting

| Transmit UPC-A check character (default) |  |
| --- | --- |
|  | Do not Transmit UPC-A check character |

### Transcoding settings

| Convert UPC-A to EAN13 (default) |  |
| --- | --- |
|  | Do not Convert UPC-A to EAN13 |

## UPC-E Settings

### Turn on/off settings

| UPC-E on (default) |  |
| --- | --- |
|  | UPC-E Off |

### Check Character setting

| Transmit UPC-E Check Character (default) |  |
| --- | --- |
|  | Do no Transmit UPC-E transmission Check Character |

### Transcoding settings

| Convert UPC-E to UPC-A |  |
| --- | --- |
|  | Do not Convert UPC-E to UPC-A (default) |

## Code 39 Settings

### Turn on/off settings

| Code 39 on (default) |  |
| --- | --- |
|  | Code 39 Off |

### Check Character settings

| Enable Code 39 Check Character |  |
| --- | --- |
|  | Disable Code 39 Check Character (default) |
| Transmit Code 39 Check Character |  |
|  | Do not Transmit Code 39 Check Character (default) |

### Start and stop character settings

| Transmit Code 39 start and stop characters |  |
| --- | --- |
|  | Do not Transmit Code 39 start and stop characters (default) |

### Code39 Full ASCII

| On |  |
| --- | --- |
|  | Off (default) |

### Code 39 Length Setting

Set the minimum and maximum length of Code 39 decoding, the value range is 4~255.

| ATT | Note: If the maximum length is less than the minimum length, only the barcode of these two lengths will be Scan. If the maximum length is equal to the minimum length, only this length is supported. |  |  |
| --- | --- | --- | --- |
| Minimum length setting |  |  |  |
|  |  |  | Maximum length setting |
| Example |  | Set the scanner to Scan only barcodes with a minimum of 8 bytes and a maximum of 12 bytes. 1. Scan "Startup Settings" 2. Scan "Minimum length setting" 3. Scan Digital Code "8" (see Appendix 1 for data and edit barcode) 4. Scan "Save" (see Appendix 1 to save or cancel) 5. Scan "Maximum length setting" 6. Scan Digital Code "1" "2" (see Appendix 1) 7. Scan "Save" (see Appendix 1 to save or cancel) 8. Scan "Exit Settings" |  |

## Code 32 settings

### Convert Code 39 to Code 32

When Code 32 is Off alone, the code output will be Code 39, and if Code 32 needs to be Off, Code 39 needs to be Off at the same time. Code 39.

| Convert Code39 to Code32 |  |
| --- | --- |
|  | Do not Convert Code39 to Code32 (default) |

## Code 128 Settings

### Turn on/off settings

| Code 128 on (default) |  |
| --- | --- |
|  | Code 128 Off |

### Check Character settings

| Enable Code 128 Check Character (default) |  |
| --- | --- |
|  | Disable Code 128 Check Character |
| Transmit Code 128 Check Character |  |
|  | Do not Transmit Code 128 Check Character (default) |

### Code 128 Length Setting

Set the minimum and maximum length of Code 128 code decoding, the value range is 4~255.

| ATT | Note: If the maximum length is less than the minimum length, only the barcode of these two lengths will be Scan. If the maximum length is equal to the minimum length, only this length is supported. |  |
| --- | --- | --- |
| Minimum length setting |  |  |
|  |  | Maximum length setting |
| Example | Set the scanner to Scan only barcodes with a minimum of 8 bytes and a maximum of 12 bytes. 1. Scan "Startup Settings" 2. Scan "Minimum length setting" 3. Scan Digital Code "8" (see Appendix 1 for data and edit barcode) 4. Scan "Save" (see Appendix 1 to save or cancel) 5. Scan "Maximum length setting" 6. Scan Digital Code "1" "2" (see Appendix 1) 7. Scan "Save" (see Appendix 1 to save or cancel) 8. Scan "Exit Settings" |  |

## Code 93 Settings

### Turn on/off settings

| Code 93 On (default) |  |
| --- | --- |
|  | Code 93 Off |

### Check Character settings

| Enable Code 93 Check Character (default) |  |
| --- | --- |
|  | Disable Code 93 Check Character |
| Transmit Code 93 check digit |  |
|  | Do not Transmit Code 93 Check Digit (default) |

### Code 93 Length Setting

Set the minimum and maximum length of Code 93 code decoding, the value range is 4~255.

| ATT | Note: If the maximum length is less than the minimum length, only the barcode of these two lengths will be Scan. If the maximum length is equal to the minimum length, only this length is supported. |  |
| --- | --- | --- |
| Minimum length setting |  |  |
|  |  | Maximum length setting |
| Example | Set the scanner to Scan only barcodes with a minimum of 8 bytes and a maximum of 12 bytes. 1. Scan "Startup Settings" 2. Scan "Minimum length setting" 3. Scan Digital Code "8" (see Appendix 1 for data and edit barcode) 4. Scan "Save" (see Appendix 1 to save or cancel) 5. Scan "Maximum length setting" 6. Scan Digital Code "1" "2" (see Appendix 1) 7. Scan "Save" (see Appendix 1 to save or cancel) 8. Scan "Exit Settings" |  |

## Codabar Settings

### Turn on/off settings

| Codabar on (default) |  |
| --- | --- |
|  | Codabar Off |

### Start and stop character settings

| Transmit Codabar start and stop characters |  |
| --- | --- |
|  | Do not Transmit Codabar start and stop characters (default) |

### Codabar length setting

Set the minimum length and maximum length of Codabar code decoding, the range of values is 4~255.

| ATT | Note: If the maximum length is less than the minimum length, only the barcode of these two lengths will be Scan. If the maximum length is equal to the minimum length, only this length is supported. |  |
| --- | --- | --- |
| Minimum length setting |  |  |
|  |  | Maximum length setting |
| Example | Set the scanner to Scan only barcodes with a minimum of 8 bytes and a maximum of 12 bytes. 1. Scan "Startup Settings" barcode 2. Scan "Minimum length setting" barcode 3. Scan Digital Code "8" (see Appendix 1 for data and edit barcode) 4. Scan "Save" (see Appendix 1 to save or cancel) 5. Scan "Maximum length setting" 6. Scan Digital Code "1" "2" (see Appendix 1) 7. Scan "Save" (see Appendix 1 to save or cancel) 8. Scan "Exit Settings" |  |

## Interleave 2 of 5 (ITF5) setting

### Turn on/off settings

| ITF5 on (default) |  |
| --- | --- |
|  | ITF5 off |

### Check Character settings

| Enable ITF25 check character (default) |  |
| --- | --- |
|  | Diable ITF25 Check Character |
| Transmit ITF25 Check Character |  |
|  | Do not Transmit ITF25 Check Charcter (default) |

### ITF25 length setting

Set the minimum length and maximum length of cross 25 code decoding, the value range is 4~255.

| ATT | Note: If the maximum length is less than the minimum length, only the barcode of these two lengths will be Scan. If the maximum length is equal to the minimum length, only this length is supported. |  |
| --- | --- | --- |
| Minimum length setting |  |  |
|  |  | Maximum length setting |
| Example | Set the scanner to Scan only barcodes with a minimum of 8 bytes and a maximum of 12 bytes. 1. Scan "Startup Settings" 2. Scan "Minimum length setting" 3. Scan Digital Code "8" (see Appendix 1 for data and edit barcode) 4. Scan "Save" (see Appendix 1 to save or cancel) 5. Scan "Maximum length setting" 6. Scan Digital Code "1" "2" (see Appendix 1) 7. Scan "Save" (see Appendix 1 to save or cancel) 8. Scan "Exit Settings" |  |

# Appendix

## Appendix 1 Data code table

|  | 0 |
| --- | --- |
| 1 |  |
|  | 2 |
| 3 |  |
|  | 4 |
| 5 |  |
|  | 6 |
| 7 |  |
|  | 8 |
| 9 |  |

|  | A |
| --- | --- |
| B |  |
|  | C |
| D |  |
|  | E |
| F |  |
|  | Cancels a string of data that was read earlier |
| Cancel current settings |  |
|  | Cancel the previous read data |
| Save |  |

## Appendix 2 Code system support list

| Code type | CODE ID |  |
| --- | --- | --- |
|  | HEX | Corresponding characters |
| Codabar | 0x42 | B |
| Code 39 | 0x62 | b |
| Code 93 | 0x69 | i |
| Code 128 | 0x6A | j |
| EAN |  |  |
| EAN-8 | 0x44 | D |
| EAN-13 | 0x64 | d |
| ISBN-10 | 0x73 | s |
| ISBN-13 | 0x53 | S |
| ISSN | None | None |
| UPC |  |  |
| UPC-A | 0x63 | c |
| UPC-E | 0x45 | E |
| 2 of 5 |  |  |
| Interleaved 2 of 5 | 0x65 | e |

## Appendix 3 Control Characters Table

| Character | Character Name | Value (decimal) | Value (hexadecimal) | Control character output Control + ASCII mode | Control character output ALT+Keypad mode |
| --- | --- | --- | --- | --- | --- |
| NUL (Null char.) | Zero Characters | 0 | 0x00 | Ctrl+@ | ALT+0 |
| SOH (Start of Header) | Title Start | 1 | 0x01 | Ctrl+A | ALT+1 |
| STX (Start of Text) | Text Start | 2 | 0x02 | Ctrl+B | ALT+2 |
| ETX (End of Text) | End of text | 3 | 0x03 | Ctrl+C | ALT+3 |
| EOT (End of Transmission) | End of transmission | 4 | 0x04 | Ctrl+D | ALT+4 |
| ENQ (Enquiry) | Inquiry | 5 | 0x05 | Ctrl+E | ALT+5 |
| ACK (Acknowledgment) | Confirmation | 6 | 0x06 | Ctrl+F | ALT+6 |
| BEL (Bell) | Ringer | 7 | 0x07 | Ctrl+G | ALT+7 |
| BS (Backspace) | Backspace | 8 | 0x08 | Ctrl+H | ALT+8 |
| HT (Horizontal Tab) | Horizontal Tabs | 9 | 0x09 | Ctrl+I | ALT+9 |
| LF (Line Feed) | line feed | 10 | 0x0a | Ctrl+J | ALT+10 |
| VT (Vertical Tab) | Vertical tabs | 11 | 0x0b | Ctrl+K | ALT+11 |
| FF (Form Feed) | Format feed to | 12 | 0x0c | Ctrl+L | ALT+12 |
| CR (Carriage Return) | Carriage return | 13 | 0x0d | Ctrl+M | ALT+13 |
| SO (Shift Out) | Move out | 14 | 0x0e | Ctrl+N | ALT+14 |
| SI (Shift In) | Move in | 15 | 0x0f | Ctrl+O | ALT+15 |
| DLE (Data Link Escape) | Data transfer code change | 16 | 0x10 | Ctrl+P | ALT+16 |
| DC1 (XON) (Device Control 1) | Device Control 1 (XON) | 17 | 0x11 | Ctrl+Q | ALT+17 |
| DC2 (Device Control 2) | Equipment Control 2 | 18 | 0x12 | Ctrl+R | ALT+18 |
| DC3 (XOFF) (Device Control 3) | Device Control 3 (XOFF) | 19 | 0x13 | Ctrl+S | ALT+19 |
| DC4 (Device Control 4) | Equipment Control 4 | 20 | 0x14 | Ctrl+T | ALT+20 |
| NAK (Negative Acknowledgement) | Negative characters | 21 | 0x15 | Ctrl+U | ALT+21 |
| SYN (Synchronous Idle) | Synchronized idle | 22 | 0x16 | Ctrl+V | ALT+22 |
| ETB (End of Trans. Block) | End transmission word group | 23 | 0x17 | Ctrl+W | ALT+23 |
| CAN (Cancel) | Cancellation | 24 | 0x18 | Ctrl+X | ALT+24 |
| EM (End of Medium) | End of Media | 25 | 0x19 | Ctrl+Y | ALT+25 |
| SUB (Substitute) | Alternative | 26 | 0x1a | Ctrl+Z | ALT+26 |
| ESC (Escape) | Exit | 27 | 0x1b | Ctrl+[ | ALT+27 |
| FS (File Separator) | File separator | 28 | 0x1c | Ctrl+\ | ALT+28 |
| GS (Group Separator) | grouping symbol | 29 | 0x1d | Ctrl+] | ALT+29 |
| RS (Request to Send) | Record separation symbols | 30 | 0x1e | Ctrl+6 | ALT+30 |
| US (Unit Separator) | Cell separator | 31 | 0x1f | Ctrl+- | ALT+31 |

## Appendix 4 ASCII code table

(Light yellow background for control characters, white background for displayable characters.)

| Binary | Decimal | Hexadecimal | Characters/abbreviations | Explanation |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | NUL (NULL) | null character |
| 1 | 1 | 1 | SOH (Start Of Headling) | Title Start |
| 10 | 2 | 2 | STX (Start Of Text) | Start of main article |
| 11 | 3 | 3 | ETX (End Of Text) | End of text |
| 100 | 4 | 4 | EOT (End Of Transmission) | End of transmission |
| 101 | 5 | 5 | ENQ (Enquiry) | Request |
| 110 | 6 | 6 | ACK (Acknowledge) | Response/response/notification received |
| 111 | 7 | 7 | BEL (Bell) | Ringer |
| 1000 | 8 | 8 | BS (Backspace) | Backspace |
| 1001 | 9 | 9 | HT (Horizontal Tab) | Horizontal Tabs |
| 1010 | 10 | 0A | LF/NL(Line Feed/New Line) | Line feed key |
| 1011 | 11 | 0B | VT (Vertical Tab) | Vertical Tabs |
| 1100 | 12 | 0C | FF/NP (Form Feed/New Page) | Page change key |
| 1101 | 13 | 0D | CR (Carriage Return) | Enter key |
| 1110 | 14 | 0E | SO (Shift Out) | No need to switch |
| 1111 | 15 | 0F | SI (Shift In) | Enable switching |
| 10000 | 16 | 10 | DLE (Data Link Escape) | Data Link Escape |
| 10001 | 17 | 11 | DC1/XON | Device Control 1/ Transmission Start |
|  |  |  | (Device Control 1/Transmission On) |  |
| 10010 | 18 | 12 | DC2 (Device Control 2) | Equipment Control 2 |
| 10011 | 19 | 13 | DC3/XOFF | Device control 3/transmission interruption |
|  |  |  | (Device Control 3/Transmission Off) |  |
| 10100 | 20 | 14 | DC4 (Device Control 4) | Equipment Control 4 |
| 10101 | 21 | 15 | NAK (Negative Acknowledge) | No response/abnormal response/rejected |
| 10110 | 22 | 16 | SYN (Synchronous Idle) | Synchronized idle |
| 10111 | 23 | 17 | ETB (End of Transmission Block) | End of block transfer/block transfer termination |
| 11000 | 24 | 18 | CAN (Cancel) | Cancellation |
| 11001 | 25 | 19 | EM (End of Medium) | End of Media / Media Storage Full / Media Break |
| 11010 | 26 | 1A | SUB (Substitute) | Substitute / Replacement |
| 11011 | 27 | 1B | ESC (Escape) | Escape/Cancel |
| 11100 | 28 | 1C | FS (File Separator) | File splitter |
| 11101 | 29 | 1D | GS (Group Separator) | Group separator / grouping character |
| 11110 | 30 | 1E | RS (Record Separator) | Record separator |
| 11111 | 31 | 1F | US (Unit Separator) | Cell separator |
| 100000 | 32 | 20 | (Space) | Space |
| 100001 | 33 | 21 | ! |  |
| 100010 | 34 | 22 | " |  |
| 100011 | 35 | 23 | # |  |
| 100100 | 36 | 24 | $ |  |
| 100101 | 37 | 25 | % |  |
| 100110 | 38 | 26 | & |  |
| 100111 | 39 | 27 | ' |  |
| 101000 | 40 | 28 | ( |  |
| 101001 | 41 | 29 | ) |  |
| 101010 | 42 | 2A | * |  |
| 101011 | 43 | 2B | + |  |
| 101100 | 44 | 2C | , |  |
| 101101 | 45 | 2D | - |  |
| 101110 | 46 | 2E | . |  |
| 101111 | 47 | 2F | / |  |
| 110000 | 48 | 30 | 0 |  |
| 110001 | 49 | 31 | 1 |  |
| 110010 | 50 | 32 | 2 |  |
| 110011 | 51 | 33 | 3 |  |
| 110100 | 52 | 34 | 4 |  |
| 110101 | 53 | 35 | 5 |  |
| 110110 | 54 | 36 | 6 |  |
| 110111 | 55 | 37 | 7 |  |
| 111000 | 56 | 38 | 8 |  |
| 111001 | 57 | 39 | 9 |  |
| 111010 | 58 | 3A | : |  |
| 111011 | 59 | 3B | ; |  |
| 111100 | 60 | 3C | < |  |
| 111101 | 61 | 3D | = |  |
| 111110 | 62 | 3E | > |  |
| 111111 | 63 | 3F | ? |  |
| 1,000,000 | 64 | 40 | @ |  |
| 1000001 | 65 | 41 | A |  |
| 1000010 | 66 | 42 | B |  |
| 1000011 | 67 | 43 | C |  |
| 1000100 | 68 | 44 | D |  |
| 1000101 | 69 | 45 | E |  |
| 1000110 | 70 | 46 | F |  |
| 1000111 | 71 | 47 | G |  |
| 1001000 | 72 | 48 | H |  |
| 1001001 | 73 | 49 | I |  |
| 1001010 | 74 | 4A | J |  |
| 1001011 | 75 | 4B | K |  |
| 1001100 | 76 | 4C | L |  |
| 1001101 | 77 | 4D | M |  |
| 1001110 | 78 | 4E | N |  |
| 1001111 | 79 | 4F | O |  |
| 1010000 | 80 | 50 | P |  |
| 1010001 | 81 | 51 | Q |  |
| 1010010 | 82 | 52 | R |  |
| 1010011 | 83 | 53 | S |  |
| 1010100 | 84 | 54 | T |  |
| 1010101 | 85 | 55 | U |  |
| 1010110 | 86 | 56 | V |  |
| 1010111 | 87 | 57 | W |  |
| 1011000 | 88 | 58 | X |  |
| 1011001 | 89 | 59 | Y |  |
| 1011010 | 90 | 5A | Z |  |
| 1011011 | 91 | 5B | [ |  |
| 1011100 | 92 | 5C | \ |  |
| 1011101 | 93 | 5D | ] |  |
| 1011110 | 94 | 5E | ^ |  |
| 1011111 | 95 | 5F | _ |  |
| 1100000 | 96 | 60 | ` |  |
| 1100001 | 97 | 61 | a |  |
| 1100010 | 98 | 62 | b |  |
| 1100011 | 99 | 63 | c |  |
| 1100100 | 100 | 64 | d |  |
| 1100101 | 101 | 65 | e |  |
| 1100110 | 102 | 66 | f |  |
| 1100111 | 103 | 67 | g |  |
| 1101000 | 104 | 68 | h |  |
| 1101001 | 105 | 69 | i |  |
| 1101010 | 106 | 6A | j |  |
| 1101011 | 107 | 6B | k |  |
| 1101100 | 108 | 6C | l |  |
| 1101101 | 109 | 6D | m |  |
| 1101110 | 110 | 6E | n |  |
| 1101111 | 111 | 6F | o |  |
| 1110000 | 112 | 70 | p |  |
| 1110001 | 113 | 71 | q |  |
| 1110010 | 114 | 72 | r |  |
| 1110011 | 115 | 73 | s |  |
| 1110100 | 116 | 74 | t |  |
| 1110101 | 117 | 75 | u |  |
| 1110110 | 118 | 76 | v |  |
| 1110111 | 119 | 77 | w |  |
| 1111000 | 120 | 78 | x |  |
| 1111001 | 121 | 79 | y |  |
| 1111010 | 122 | 7A | z |  |
| 1111011 | 123 | 7B | { |  |
| 1111100 | 124 | 7C | ¦ |  |
| 1111101 | 125 | 7D | } |  |
| 1111110 | 126 | 7E | ~ |  |
| 1111111 | 127 | 7F | DEL (Delete) | Delete |